🐦‍⬛

Animatronic Bird Controller

v4  —  Multi-Prop Edition

Captive Portal Setup  ·  Runtime Configuration  ·  Location / Room / Device MQTT Hierarchy

# What's New in v4

v4 adds first-time WiFi/MQTT setup via captive portal, full runtime device configuration stored in config.json, and a three-level MQTT topic hierarchy designed for environments with many props across multiple rooms.

| Feature | Description |
| --- | --- |
| Captive portal setup | On first boot the ESP32 becomes a WiFi AP called BirdSetup. Connect any phone — an automatic popup loads the setup page. |
| Runtime configuration | WiFi, MQTT, and identity live in config.json on SPIFFS — no reflashing ever needed. |
| 3-level MQTT topics | location/room/device hierarchy (e.g. props/tiki_bar/parrot1). Supports MQTT + wildcard broadcasting. |
| Settings tab | Edit all config from the browser. Auto-reboots when WiFi/MQTT changes. |
| Reset to AP mode | One button wipes config.json and restarts as BirdSetup for reconfiguration. |
| Identity announcement | On MQTT connect each bird publishes its name, hostname, IP, and prefix so show controllers can discover all props. |
| Multi-prop ready | Flash the same binary to every bird. Configure each via the portal. One MQTT message controls whole rooms. |

▶  Upgrading from v3? The only breaking change is that WiFi and MQTT credentials are no longer in raven_config.h. On first boot v4 launches the setup AP — configure once, then it behaves exactly like v3.

# Multi-Prop Setup

Flash the same firmware binary to every bird. Configure each individually through the captive portal.

| Bird | Location | Room | Device ID | Full MQTT prefix |
| --- | --- | --- | --- | --- |
| Raven 1 | props | study | raven1 | props/study/raven1 |
| Raven 2 | props | study | raven2 | props/study/raven2 |
| Parrot 1 | props | tiki_bar | parrot1 | props/tiki_bar/parrot1 |
| Parrot 2 | props | tiki_bar | parrot2 | props/tiki_bar/parrot2 |
| Parrot 3 | props | tiki_bar | parrot3 | props/tiki_bar/parrot3 |

MQTT Wildcard Broadcasting

MQTT + wildcards let a show controller target whole rooms with a single message:

# One specific bird props/study/raven1/sequence # All parrots in the tiki bar simultaneously props/tiki_bar/+/sequence # Every bird in the entire show props/+/+/sequence # Discover all birds at startup props/+/+/query → payload: all

# First-Time Setup — Captive Portal

Step 1 — Flash the firmware

pio run --target uploadfs --environment esp32dev   # filesystem first

pio run --target upload   --environment esp32dev   # then firmware

Step 2 — Connect to BirdSetup AP

On your phone or laptop, open WiFi settings and connect to the BirdSetup network. A setup page should pop up automatically. If it doesn't, navigate to http://192.168.4.1

Step 3 — Fill in the setup form

| Field | Example | Notes |
| --- | --- | --- |
| Device Name | Raven 1 | Display label in the web UI header |
| Hostname | raven1 | Used for http://raven1.local — lowercase, no spaces |
| Location | props | Top level of MQTT hierarchy |
| Room | tiki_bar | Middle level — group of props in a space |
| Device ID | parrot1 | Bottom level — unique per bird |
| MQTT Broker | homeassistant.local | Hostname or IP of your broker |
| MQTT Port | 1883 | Standard MQTT port |
| WiFi SSID | MyNetwork | Your WiFi network name |
| WiFi Password | •••••••• | Your WiFi password |

Step 4 — Save & Connect

Click Save & Connect. The device saves config.json and reboots. In 10–15 seconds it joins your WiFi network and is reachable at http://[hostname].local (e.g. http://raven1.local).

▶  Repeat Steps 2–4 for each bird. Each takes about 2 minutes. Configure them one at a time — they all share the same AP name.

# Hardware

## Raven Mechanic Kit

Mr. Chicken's Prop Shop — Animatronic Raven Kit

https://chickenprops.com/products/animatronic-raven-kit

Available in Basic (mechanical parts only), Deluxe (Basic + servos), and Bare Bones configurations.

## Servos

| Qty | Part | Role |
| --- | --- | --- |
| 1× | Hitec HS-53 | Beak open / close |
| 3× | Hitec HS-425BB | Head pan, head tilt, body bob |
| 1× | Hitec HS-645MG | Wings (left and right) |

## Electronics

| Component | Part | Notes |
| --- | --- | --- |
| Microcontroller | ELEGOO ESP32 Dev Board — 38-pin WROOM, CP2102 | Any esp32dev board works |
| Servo Controller | Lynxmotion SSC-32U | USB + full-duplex UART |
| Audio Amplifier | Adafruit MAX98357A I2S Breakout — PID: 3006 | 3W Class D, I2S input |
| Speaker | Adafruit Mono Enclosed Speaker 3W 4Ω — PID: 4445 | Bare wire leads to OUT+/OUT− |
| Servo PSU | Mean Well GST60A07-P1J — 7.5V 5A | Trim SSC-32U VS+ to 6V if needed |
| Logic PSU | Mean Well GST25A05-P1J — 5V 4A | Powers ESP32 + MAX98357A |

▶  Both Mean Well GST units have an IEC C14 inlet — they require an IEC C13 power cord which is not included.

# Power Architecture

Three separate power domains — keep servo and logic power isolated. Common ground tie between SSC-32U and ESP32 is required for UART communication.

| Power Supply | Connects to | Notes |
| --- | --- | --- |
| Mean Well GST60A07-P1J 7.5V 5A | SSC-32U VS+ / GND | Servo power only — trim to 6V via SSC-32U trimmer |
| Mean Well GST25A05-P1J 5V 4A | ESP32 VIN / GND | Logic power |
| (same 5V supply) | MAX98357A VIN / GND | Audio power |
| Common ground tie | SSC-32U GND → ESP32 GND | CRITICAL — UART will not work without this |

# Wiring

## ESP32 → SSC-32U (UART Serial)

| ESP32 Pin | SSC-32U Pin | Notes |
| --- | --- | --- |
| GPIO 22 (TX) | RX | Commands to servo controller |
| GPIO 23 (RX) | TX | Done signal back to ESP32 |
| GND | GND | Common ground — required |

## ESP32 → MAX98357A (I2S Audio)

| ESP32 Pin | MAX98357A Pin | Notes |
| --- | --- | --- |
| GPIO 27 | BCLK | Bit clock |
| GPIO 14 | LRC | Left/Right clock |
| GPIO 13 | DIN | Serial audio data |
| 3.3V | SD_MODE | Tie high — amplifier always on |
| 5V | VIN | Power |
| GND | GND | Common ground |

▶  GAIN pin: Float = 9dB (default)  |  GND = 12dB  |  3.3V = 6dB

## SSC-32U Servo Channel Map

| Channel | Servo | Movement |
| --- | --- | --- |
| CH 0 | Hitec HS-53 | Beak open / close |
| CH 1 | Hitec HS-425BB | Head pan — left / right |
| CH 2 | Hitec HS-425BB | Head tilt — up / down |
| CH 3 | Hitec HS-645MG | Left wing |
| CH 4 | Hitec HS-645MG | Right wing |
| CH 5 | Hitec HS-425BB | Body bob |

# MQTT Topics

All topics are relative to the device's configured prefix (e.g. props/study/raven1).

## Subscribe — Send to Bird

| Leaf topic | Example payload | Effect |
| --- | --- | --- |
| /command | beak_open, head_left, neutral… | Single named move |
| /sequence | caw, alert, wingflap, idle, sleep | Built-in animation sequence |
| /sequence | my_sequence_name | Saved keyframe sequence |
| /sound | caw | Play sound + bound sequence |
| /audio | caw | Audio only — no sequence |
| /servo | 0 1800 300 | Raw: channel, µs, time ms |
| /query | all / sounds / sequences / identity | Request data publish |

## Publish — Bird Sends

| Leaf topic | Payload | When |
| --- | --- | --- |
| /status | Status string | After every action |
| /identity | JSON: name, hostname, ip, prefix | On MQTT connect + identity query |
| /sounds/list | JSON sound library | On connect + query |
| /sequences/list | JSON list of sequence names | On connect + query |

# Web Interface

## Control

Sequences, head D-pad, beak/wings, body bob, raw servo sliders, and custom keyframe sequences. The live indicator pill shows the currently running sequence.

## Sounds

Upload WAV files via drag-and-drop. Assign bound sequences (triggered automatically when a sound plays). Manage SPIFFS storage with a live usage bar.

## Calibrate

Tune every named servo position live — the Test button moves the servo immediately so you can dial in exact positions. Save All persists to positions.json.

## Sequences

Keyframe editor — pose the bird using the servo sliders, click Add Keyframe, repeat, name the sequence, and save. Saved sequences appear immediately in the Control tab and are playable via MQTT.

## Settings  (new in v4)

Edit all device configuration post-setup without the portal: device name, hostname, MQTT location / room / device (shows the built prefix live), MQTT broker and port, WiFi credentials.

- WiFi or MQTT changes trigger an automatic reboot to reconnect

- Reset to Setup AP wipes config.json and reboots as BirdSetup

- Password field shows •••••••• — only replaced if you type a new one

# File Structure

| File | Purpose |
| --- | --- |
| platformio.ini | Build config — board: esp32dev |
| data/index.html | Web UI — all 5 tabs in a single file |
| data/sounds.json | Sound library — file/label/sequence entries |
| data/positions.json | Calibrated servo positions in µs |
| data/sequences/ | Saved keyframe sequences — one JSON file per sequence |
| include/raven_config.h | Hardware pins and compile-time defaults only — no credentials |
| include/raven_device.h | Runtime config — loads config.json, provides device identity and MQTT prefix |
| include/raven_captive.h | Captive portal — AP mode, DNS server, setup page HTML, /save endpoint |
| include/raven_webui.h | ESPAsyncWebServer, WebSocket, full HTTP API including /api/config |
| include/raven_mqtt.h | MQTT client — runtime topic prefix, identity announcement |
| include/raven_mdns.h | mDNS — runtime hostname from device config |
| src/main.cpp | Boot flow: load config → captive portal check → WiFi → full init |
| (SPIFFS) config.json | Created by the captive portal — never shipped in the zip |

# Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| BirdSetup AP doesn't appear | Firmware not flashed | Check serial output, reflash |
| Setup popup doesn't appear | Captive portal not triggered | Navigate to http://192.168.4.1 manually |
| raven1.local not found | mDNS blocked on network | Use IP address shown in serial output |
| Servos not responding | No UART comms | Check GPIO 22/23, SSC-32U baud 9600 |
| Servos jitter / ESP resets | Servo PSU on ESP32 rail | Isolate servo power to SSC-32U VS+ only |
| No audio | I2S wiring | Check GPIO 27/14/13, SD_MODE tied to 3.3V |
| MQTT not connecting | Wrong broker / port | Check Settings tab, verify broker reachable |
| Need to change WiFi after setup | — | Settings tab → update SSID/password → Save |
| Completely stuck / wrong config | — | Settings tab → Reset to Setup AP |

# Resources

- Mr. Chicken's Build Resources — drive.google.com/drive/u/0/folders/1Ht4HYf0q8vE1VhdCadyyANCFJWSVyEcL

- Raven Builders Facebook Group — facebook.com/groups/185149192169022

- Lynxmotion SSC-32U Docs — lynxmotion.com/c-153-ssc-32u.aspx

- Adafruit MAX98357A Guide — learn.adafruit.com/adafruit-max98357-i2s-class-d-mono-amp

- Mean Well GST60A07-P1J Datasheet — meanwell.com

- PlatformIO ESP32 Reference — docs.platformio.org

Controller firmware developed with Claude (Anthropic)  ·  Raven mechanic kit by Mr. Chicken's Prop Shop / Jasper J. Anderson FX